A. Laboratory Equipment Dept. of Food Processing Technology:

- Tensile Strength Tester: Microprocessor based digital model, Capacity: 5-200KGF, Testing Speed: 12+/-3 MM/Min

- Impact Tester

- Bursting Strength Tester: Automatic digital microprocessor based 2-in-1 model for accurate testing of materials with lower burst value as well higher burst value,1-70 Kg/Cm2

- Tear Resistance Tester: Elmendorf type Tear Resistance Tester., Capacity:400KGF

- Cobb Tester

- Edge Crush Tester: Microprocessor based digital model with attachments for flat, edge & ring crush test. Capacity: 500Kg.

- Pin Hole Detector: Table Top model, self illuminated chamber with one side Glass Top and Digital Counter.

- Ultra Pure Water Purification System: Imported to produce up to 0.6 L/Min with integral pump, fully automatic system, Resistancy: 18.2MΩ.cm at 250 C, TOC<10, Particulates:<0.22 µm, with 10L reservoir attached.

- Deep Freezer: Imported, capacity:256 L/220L (Inner), upright freeze, Temperature:-200 C, Re-circulated air cooling, frost free drawer system, auto defrost.

- Vacuum Packaging machine: Table top model, vacuum pressure controller and indicator.

- Baking Oven: 2 and 3 deck, temperature controller and timer for each deck.

- Cake mixer: Planetary/spiral, Bowl can be tilted upside down, with temperature and speed controller, timer, 5Kg cap.

- Dough Kneader: Bowl can be tilted upside down with temperature and speed controller, timer, 5Kg cap.

- Vacuum Pump: Oil free, max. vacuum:600mm, Hg, max. pressure: 25 psi, motor rotation: 1450 rpm.

- Can reforming unit: Electrical, for all can diameter.

- Can flanging machine: For all can diameters.

B. Equipment Project work, Dept. of Physics

- Magnetron: Pulsed co axial, X-band, power greater than 80kW, Air cooled.

- Power supply compatible with the Magnetron: Include the current, voltage and mounting arrangement for the magnetron (Highly stabilized and shielded)

- Power meter: In X–band (Measurement of high power)

- Sensor: Compatible with the power.

- Accessories: High power circulators, wave guides with choke flanges, high power loads, adapters(HP), frequency meter, connectors, high precision attenuators etc.(Capable in handling high power in X-band range)

- Electromagnet with Gauss meter: Uniform about pole area of 6CM2, ltesla at pole distance of 2CM.

- Power Meter: Upto atleast 18GHz (Agilent-HP)

- Sensor: Average power compatible with power meter.(Agilent-HP)

C. Equipment-Dept. of Mechanical Engineering.

- CNC PRODUCTION LATHE:

Specifications:

Make: MTAB

Capacity:

Maximum Turning Diameter			160mm

Maximum Boring Length				20mm

Distance between centre				250mm

Max. turning length				130mm

Bar capacity (diameter)				25mm

Traverse:

Travel on X-axis					100mm

Travel on Z-axis					250mm

Rapid Traverse X-axis				10m/min

Rapid Traverse Z-axis				10m/min

Swing over bed					220mm

Spindle motor					5hp

The machine should be FMS compatible.

Turret:

8 stations/programmable turret

Maximum Boring Bar Dia			20mm

Tool Cross section:				12x12

CNC Control Options:

Fanuc OiT Mate with servo Motors

Siemens 8020D Controller

- ARC WELDING MACHINE:

Specifications:

Primary voltage (volt)				380/415

Phase						1-3

Rating (KVA)

Continuous					7-30

Maximum						13-45

Open circuit voltage (volt)			50-100

Open arc voltage (volt)				20-45

Welding current range (ampere)			30-600

Welding electrode range (mm)

Minimum						1.5-2.5

Maximum						4-6

Cooling						air

3. UNIVERSAL MILLING MACHINE: ALONG WITH ALL STANDARD ACCESSORIES

Make:  HMT/BATLIBOI/BFW

# Specification

| specifications | UNIT |  |
| --- | --- | --- |
| Table width | mm | 400 |
| Table length | mm | 1400 |
| Longitudinal Traverse | mm | 1000 |
| Ram Traverse | mm | 450 |
| Vertical Traverse | mm | 450 |
| Feed rate longitudinal axis | mm/min | 10-1800 |
| Feed rate cross axis | mm/min | 10-1800 |
| Feed rate vertical axis | mm/min | 2.5-450 |
| Rapid rate longitudinal axis | m/min | 4 |
| Rapid rate cross axis | m/min | 4 |
| Rapid rate vertical axis | m/min | 1 |
| Spindle power | kw | 11 |
| Spindle speeds | rpm | 40-2000 |
| Spindle taper |  | ISO 40/ISO50 |

SPECIAL ACCESSORIES FOR UNIVERSAL MILLING MACHINE:

- Coolant equipment

- Machine lamp

- Universal Milling Head

- Vertical Milling Head type B-3501-3

- Slotting attachment type B-2215-s

- rack milling attachment type B-2215-Z

- Long milling arbor Set D ISO 40       27  500 mm

32  500 mm

- Stub Arbor with key ISO 40 		22  21 mm

27  21 mm

32  21 mm

- Collect adoopter ISO 40

- Set of collect chuck (Metric) (3-32 mm)

- Reduction sleeve tang tytpe MT2

- Reduction sleeve screw type MT2

- Precision machine vice with swivel base 160 mm

- Self centring vice Type B-927 (10-80 mm)

- Universal dividing hrad type B-2211T

- Circular table with power feed 400 mm

4. HORIZONTAL MILLING MACHINE:  ALONG WITH ALL STANDARDACCESSORIES

Make: BFW/ BATLIBOI/HMT

# Specification

| Table Size | 1350 x 320 |
| --- | --- |
| T-slots No./ Width/ CD. mm | 3/ 14/ 63 |
| Traverses Longitudinal mm | 800 |
| Cross mm | 260 |
| Vertical mm | 390/400 |
| Milling spindle front bearing Ø mm | 85 |
| Spindle taper ISO | 40 |
| Speed range rpm | 45-2000 |
| No./ progression | 12/ 1.4 |
| No. of Feeds Nos. | 18 |
| Feed range (Progression 1.25) |  |
| Longitudinal mm/ min | 14-900 |
| Transverse mm/ min | 14-900 |
| Vertical mm/ min | 4-250 |
| Rapid Transverse |  |
| Longitudinal mm/ min | 4000 |
| Transverse mm/ min | 4000 |
| Vertical mm/ min | 1000 |
| Milling spindle motor kW | 5.5 |
| Feed motor kW | 1.5 |
| Max. safe weight on the table kg | 400 |
| Weight/ Nett (approx.) kg | 2200 |

SPECIAL ACCESSORIES FOR HORIZONTAL MILLING MACHINE:

- Coolant equipment

- Machine lamp

- Long arbor ISO 40   	27  500 mm

32  500 mm

- Stub arbor	22  21 mm

27  21 mm

32  21 mm

- Collect adopter ISO 40

- Set of collets (3-32) mm

- Reduction sleeve IS 5927 MT1 ISO 40

- Universal Indexing Head with self cetring chuck (Brown &  Sharp type)

- Machine vice (Fixed)

- Machine Vice (Universal)

- Rotary table

- Self Centring vice B-927 (10-80 mm)

- Set of change gear (24-100 teeth)

- Tail stock

5. VERTICAL MILLING MACHINE: ALONG WITH ALL STANDARDACCESSORIES

Make: BFW/BATLIBOI/HMT

Specification

| Overall size (L x W) mm | 1555 x 315 |
| --- | --- |
| Clamping area mm | 1350 x 315 |
| T-slots No./ Width/ CD. mm | 3/ 14/ 63 |
| Swivel range of table to both sides degrees | - |
| Longitudinal movement |  |
| Automatic mm | 800 |
| Manual mm | 830 |
| Transverse movement |  |
| Automatic mm | 230 |
| Manual mm | 260 |
| Vertical movement |  |
| Automatic, mm | 400 |
| Manual ,mm | 420 |
| # Spindle |  |
| Quill stroke, manual, mm | 70 |
| Milling spindle front bearing Ø mm | 85 |
| Spindle taper ISO | 40 |
| Vertical milling head swivelling to both side degrees | 45 |
| Speed range rpm | 45-2000 |
| No./ progression | 12/ 1.4 |
| Feed |  |
| No. of Feeds Nos. | 18 |
| Feed range (Progression 1.25) |  |
| Longitudinal mm/ min | 16-800 |
| Transverse mm/ min | 16-800 |
| Vertical mm/ min | 4-200 |
| Rapid Transverse |  |
| Longitudinal mm/ min | 4000 |
| Transverse mm/ min | 4000 |
| Vertical mm/ min | 1000 |
| Milling spindle motor kW | 5.5 |
| Feed motor kW | 1.5 |

##### SPECIAL ACCESSORIES FOR VERTICAL MILLING MACHINE:

- Coolant equipment

- Machine lamp

- Stub Arbor with key ISO 40 		22  21 mm

27  21 mm

32  21 mm

- Collect adoopter ISO 40

- Set of collect chuck (Metric) (3-32 mm)

- Reduction sleeve tang tytpe MT2

- Reduction sleeve screw type MT2

- Precision machine vice with swivel base 160 mm

- Self centring vice Type B-927 (10-80 mm)

- Universal dividing hrad type B-2211T

- Circular table with power feed 400 mm

6. SHAPING MACHINE ALONG WITH ALL STANDARD ACCESSORIES

Make: AARPEE

### Specification

| Length of the ram stroke, mm | 915 |
| --- | --- |
| Length of ram without tool slide, mm | 1740 |
| Maximum distance from table to ram, mm | 521 |
| Minimum distance from table to ram, mm | 140 |
| Horizontal traverse of the table, mm | 870 |
| Vertical traverse of the table, mm | 390 |
| Angular movement of the table on either side, degree | fixed |
| Working surface of the table, mm | 457915 |
| Maximum size of the tool shank accommodated, mm | 3821 |
| Maximum vertical travel of tool slide, mm | 200 |
| Maximum swivel of tool head, mm | 60I, 60r |
| Length of main slide, mm |  |
| Number of ram speeds |  |
| Range of ram speed, stroke per minute | 12 25 40 67 |
| Range of table feed per stroke of ram, mm | 0.229-0.687 |
| Range of tool head feed | Hand feed |
| Main drive motor, kw | 5hp |
| Speed, RPM | 960 |

SPECIAL ACCESSORIES

- Key way cutting attachment

- Machine light

- Auto tool lifter

7. SURFACE GRINDING MACHINE ALONG WITH ALL STANDARDACCESSORIES

Make: Praga Tools

### Specification

| ### Table |  |
| --- | --- |
| Work area, mm | 455 x 150 – 600 x 200 |
| Longitudinal traverse, mm | 152-650 |
| Cross traverse, mm | 101-228 |
| ### Feeds |  |
| Table longitudinal speeds, m/min | 1.5 to 18 |
| Cross feed per stroke at each reversal, mm | 0.175-1.75 |
| ### Wheel head |  |
| Vertical movement, mm | 101-305 |
| Speed of wheel, rpm | 2800-4000 |
| Size of the wheel, mm | 178 x 13 x 32 – 203 x 20 x 25 |
| ### Motors |  |
| Wheel head motor, Kw | 0.75 – 1.5 |
| Hydraulic pump motor, Kw | 0.375 |

SPECIAL ACCESSORIES

- Magnatic chuck 600  200 mm

- Diamond  holder and diamond for magnaticc chuck

- Diamond for std. Table mounted wheel truing attachment

- Diamond for Radius and angular wheel forming attachment.

- Dust exhaust equipment

- Electrical pump and coolant fitting UV-6100

- lighting equipment with low volt transformer

- static wheel balancing unit 2101-001

- universal 3 way swiveling vice admits work 65 mm width of jaw 100 mm x 13

- Sine Vice.

- 90 vice

- Grinding Wheel (Soft and hard grade)

8. PEDESTAL GRINDING MACHINE ALONG WITH ALL STANDARD ACCESSORIES

### Specification

| Size of grinding wheel, mm | 150 x 20 x 15 – 1000 x 406.4 x 200 |
| --- | --- |
| Wheel center distance, mm | 600-1500 |
| Base to the center line, mm | 810-910 |
| Diameter of spindle in bearings, mm | 35-100 |
| No of motors | 1-2 |
| Power of motor, kW | 1.5-15 |
| Speed of motor, rpm | 450-2250 |
| Number of speeds | 2-3 |
| Net weight, kg | 27-1150 |

### Special accessories:

### Grinding wheel Dresser

- Coolant equipment

### 9. UNIVERSAL TOOL AND CUTTER GRINDING MACHINE: ALONG WITH ALL STANDARD ACCESSORIES

### Make: HMT (with power rise and fall of wheel head)

| Specifications : |  |  |
| --- | --- | --- |
| Centre height | mm | 130 |
| Swing | mm | 280 |
| Swing with raising block | mm | 370 |
| Admit between tailstock centres | mm | 760 |
| Admit between work head & tailstock centre | mm | 615 |
| Max. job weight for cylindrical grinding | kg | 15 |
| Longitudinal travel of table | mm | 510 |
| Clamping area | mm | 980x140 |
| Cross traverse | mm | 250 |
| Tailstock taper |  | MT2 |
| Total power | kW | 3 |

Special Accessories:

- Indexing unit with one indexing ring having 24 Dvn

- 3-speed work head for cylindrical grinding (.18kw)

- Right hand tailstock with center

- 3-jaw chuck dia 160 mm

- Draw bar for above chuck

- Two-jaw steady

- Wheel guard with bracket

- Grinding wheel 200/20x32 A46 K5V Eqv

- Universal vice for surface grinding

- Attachment for grinding of end relief disk cutter

- Gear milling cutter grinding attachment with 3 bushes

- Centre attachment for long reamer

- Attachment for grinding carbide tipped tools

- Twist drill sharpening attachment dia 5/25 mm

- Dust exhausting device

- Water cooling device

- Wheel shoulder dressing attachment

- Radius truing attachment

- Dressing diamond 1 ct. with par. shank dia 12 x 150

- Machine lamp 220 volts (without bulb)

- Set of spare

10. HIGH SPEED PRECISION LATHE:

Make: HMT along with all standard accessories

Specifications

| Height of centers | mm | 260 |
| --- | --- | --- |
| Swing over bed | mm | 575 |
| Swing over cross slide | mm | 350 |
| Swing in gap (optional) | mm | 800 |
| Distance between centers | mm | 2000 |
| Spindle Nose / Bore | mm | A2-6* / 53 |
| Spindle Speed range | rpm | 16 from 40-2040 forward 7 from 60-1430 reverse |
| Spindle power | kW | 11 |
| Feed range (longitudinal) | mm/rev | 60 from 0.04-2.24 |
| Feed range (cross) | mm/rev | 60 from 0.02-1.12 |
| Lead screw pitch | mm | 6 |
| Metric threads | mm | 48 from 0.5-28 |
| Inch threads | tpi | 60 from 56-1 |
| Module threads | mm | 40 from 0.25-14 |
| Diametral pitch (optional) | mm | 43 from 112-2 |
| Tailstock sleeve travel | mm | 200 |
| Main motor power | kW | 7.5 (std.) / 11.0 (opt.) |

Special Accessories

- Collect chuck with collets (23 piece)

- Machine lamp for 24 volts (without lamp)

- Swarf tray coolant equipment

- Splash guard

- Star-Delta starter

- Steady rest 8-145 mm

- Change gear for cutting modules and diametral pitch (4 nos. change gear and one name plate)

- Precision face plate dia 355 mm

- 3-jaw self centring chuck dia 250 mm

- Taper turning attachment

- Hydraulic chuck with 250 mm (4 jaw)

- Hydraulic power pack

- Rotating type hydraulic cylinder OSP-120

- Multicut tracer (Manual) for C. D. 2000 mm

- Precision 3-jaw self centring chuck with hardened

- Jaw-reversible for internal & external chucking dia 250 mm

- Universal Face Plate dia 550 mm

- Follower rest 8-80 mm

- Longitudinal stop

- Stop roll

###### 11. PILLAR DRILLING MACHINE ALONG WITH ALL STANDARD ACCESSORIES

Make: EIFCO

### Specification

| Drilling Diameter in steel, mm | 30 |
| --- | --- |
| Spindle travel, mm | 220 |
| Number of Spindle speeds | 6 |
| Range of speeds, rpm | 3000 |
| Feed values, mm/rev | 1.8-2 |
| Main motor, KW | 1-1.2 |
| Work table overall dimensions, mmmm | 450400 |
| Diameter of column, mm | 300-350 |
| Max. Distance between column to spindle center, mm | 250-300 |
| Net weight, Kg | 800-1000 |
| Overall dimension(lengthwidth),mm | 800550 |

TECHNICAL DATA Unit

Accessories

1. Drill chuck with set of suitable sleeve.

# Technical Specifications:

# PRO-E Software Requirements

| Computer aided Design (CAD) |  |
| --- | --- |
|  | #### Requirement / Capability |
| ##### Software architecture |  |
| - The Software should have a single database structure. - Parametric feature based models should extend throughout all aspects of product development including modeling, analysis, drafting, and NC |  |
| ##### Transition from 2D to 3D |  |
| - The software should be capable of cleaning and organizing imported DWG file, for use in 3D design environment - The software should be capable of mapping selected 2D geometry to 3D plane automatically. - Software should be capable of generating simple features automatically after placing the appropriate 2D geometry on respective planes. - This functionality should be wizard based and easy to use |  |
| ##### 3D Geometry Creation & Manipulation |  |
| - Software should have tools like drag handles and intelligent snapping in order to reduce Menu picks and Mouse travel while designing and Modeling the Part - The Software must have solid modeling capability - It should then be possible to use the set of modeling features to create complex geometry. This geometry includes joggles, tapers, machine steps, compound contours, stiffeners, and stringers - The software should have surface creation tools like extrude, revolve, sweep, and blend for creating and manipulating complex surfaces. - The software should be capable of performing operations like scale, re-orient, taper, stretch, bend and twist geometry, Dynamically. - The Software should provide the ability to capture relationships between different features in a model. This enables the incorporation of design rules, rule constrained geometry by way of logic, laws, and equations. - The software should have dynamic feature preview capability. - The Software should support the creation of families of parts using a spread sheet interface. It should be possible to use this tool to create and maintain standard parts libraries for components like tubing and standard structural shapes. - It should be possible to manipulate model geometry using tools like copying, mirroring, translating, patterning and user defined features. It should also be possible to suppress, re-order, redefine, and delete features. All of these manipulation tools should maintain associativity. - The Software should provide full associativity between parts, assemblies and engineering drawings. Changes made in one application should be reflected in all applications. |  |
| ##### Weld Modeling and Documentation |  |
| - Software should be capable of defining joining information for welded parts and assemblies and should capable of extracting mass properties, clearance and interference considering weld joints. - The software should have facility to select different type of weld and it should display process information of each Weld type. - Software should capable of generating complete documentation in 2D drawing environment and should be associative with weld features in 3D Part/Assembly Model. |  |
| ##### Assembly Modeling |  |
| - There should be an assembly file that is distinct from the component files. This assembly file should not duplicate part geometry information. It should reference the part files. - There should be not artificial limit restricting the number of components in an assembly or the total file size. - The software should have functionality like simple drag and drop , for automated component placement - The Software should support the creation of families of assemblies using a spreadsheet interface. It should be possible to use this tool to create and maintain standard libraries of assemblies and sub-assemblies. - The Software should allow users to assemble components by establishing relationships between its geometry and the geometry of other components. To capture design intent, it should also be possible to establish dimensional relationships between parts so that changing one part in an assembly can cause other related parts to change as well. - Ability to create part geometry with the assembly referencing other part or assembly geometry. Ability to add feature geometry that references assembly geometry to a part. - Ability to replace components within the assembly with other components. Ability to simplify detailed assembly to lower level of detail using components of less detail - Ability to reuse same part or assembly multiple times within an assembly without recreating files - Ability to quickly create multiple explosions automatically - Ability to add offset lines - Ability to quickly modify explosions by dragging and dropping - Automatic tracking of BOM information in real time - Different methods to verify fit of components with assembly - Mechanism to analyze tolerance stack up in an assembly - Automatic update of changes made to components to higher level assemblies via both dimensions and geometry update - Any changes made to the assembly should automatically update any associated engineering drawings - Ability to remove a component from the assembly without requiring it to be deleted and reassembled later - The software should be capable of representing Parts or Sub-assemblies in order to minimize utilization of computer resources and to provide only required data to User while working with assemblies. - Software should be capable of designing large & complex assemblies by using Tools that supports top down design approach - Software should be capable of documenting assembly Processes |  |
| ##### Imported Data cleaning and repair |  |
| - The software should have functionality to speed up the cleaning and data fixing activities while working on imported 3D data |  |
| ##### Quality Check Tools |  |
| - The software should have functionality to check Parts, Assemblies and Drawings against pre-defined Modeling and detailing Practices and standards |  |
| ##### Mechanism Design |  |
| - Integrated Mechanism Design capabilities that could define mechanism and even convert assembly into mechanisms - Animate mechanism assemblies through interactive dragging and user-defined motions - Functionalities to detect Interference, clashes during animation and calculate interference volume at any time during the motion - Software should be capable of simulating reactions to acceleration forces & weight of moving components with the ability of incorporating springs, dampers, motors, friction, gravity and other dynamic influences |  |
| ##### Sheet Metal Modeling |  |
| - The Software should be able to create accurate solid models of all sheet metal geometry. It should provide flexible tools to create sheet metal walls and other sheet metal geometry including cuts, punches, rips, twists and forms. It should be possible to create this geometry in both the bent and flat states. - The Software should create accurate solid representations of walls including the deformation due to bends. This includes the ability to create cuts that penetrate bend regions, and bends that penetrate cut regions - The Software should provide flexible tools that enable the accurate calculation of the developed length of unbent walls. These tools should include the use of default and customized bend tables, specialized equations, and Y-factors - The Software should be able to create automatic and manual bend relief’s. - The Software should let the user specify a bend order for sheet metal parts. It should be possible to document this bend order in drawings and use the bend order to simulate the bending process. - The Software should be able to convert parts with non-uniform wall thickness (solid parts) to sheet metal parts. Similarly, the Software should be able to convert surface models into sheet metal parts with uniform wall thickness. |  |
| ##### Detailed Drawing Generation |  |
| - Any changes made to the part or assembly should automatically be updated in the drawing. This includes updating of any views as well as any annotations in the drawing - After one general view has been placed, the software should be able to create any additional projection view without specifying any orientation constraints - Mechanism to create different types of views including projection views, detailed views, partial views, etc. - Ability to quickly place and update location of views. Have scale for different views and the sheet. Display models in hidden line, wire frame. Ability to customize display of different entities in individual views. - Drawing created using the package should conform with AMSE Y14.5 - Different types of annotation should be created in the drawing. Notes, symbols, etc. should update as the geometry changes - Ability to add changeable information in a drawing in the form of notes. Information from the model should be readily accessible in the notes - Ability to have a library of notes and symbols that can be re used minimizing time for creation of similar annotation in user defined format - Have library of ANSI standard drawing symbols readily available for inclusion in drawings. Different types of symbols such as electrical, piping, heating or welding. The symbols should be user modifiable. - Create engineering drawing of whole assemblies or parts. All relevant detail should be viewable in the drawing. Drawings should also be show whole assemblies or simplified versions of the assemblies or parts on the same sheet - Ability to show as many models as needed on the same sheet. Can be parts or assemblies. - Ability to create and reuse standard drawing formats. Formats should be able to access drawing and part information to automate title block definition. - Easy to use geometric tolerancing within the drawing - Ability to create drawing tables (grid of row and columns) and include it drawing, formats and conceptual layouts. Ability to reuse tables. All attributes of the table such as number of columns and rows should be easily modifiable - Ability to customize engineering drawing environment. Options such as text parameters, arrow length and style should be controlled from a central location. Ability to create and reuse a standard setup - Mechanism to compare two similar engineering drawings for differences in dimensions or topology - Provides tools to markup a single engineering document without affecting the document - Perform markups in a collaborative environment. View markups simultaneously or individually - Tools for the addition of general drafting entities for further customization of the drawing - Ability to export and import engineering drawings via DXF, DWG, IGES, VDA and SET files |  |
| ##### Technical Publication |  |
| ##### The Software must be able to automatically generate bill of material information. - The Software should support Microsoft’s Object Linking and Embedding so that models can be pasted into OLE compliant applications. These graphics can then be used to create technical presentations and documentation. - The Software should provide an interface to the World Wide Web that enables the export of parts, assemblies, and process plans to complete Web pages using standard HTML, VRML, CGM, and JPEG formats and Java applets. Exported information can then be viewed on the Internet or an intranet using standard Web browsers |  |
| ##### Design sensitivity Study |  |
| Graphical feedback |  |
| - Software should be capable of applying multiple objectives to a design scenario and then carry out feasibility studies by creating number of virtual prototypes as needed to match the requirements - Software should deliver the results in easy to read graphical format to choose the optimum design |  |
| ##### Cabling Design |  |
| - Software should be capable of Automatically route wires and cables in 3D space and it should logically reference diagram schematic data - Software should be capable of selecting preferred terminators, plugs, - and wire strip length Automatically - Software should capable of automatically generating - Customizable BOMs, wire lists, and flattened harnesses |  |
| ##### Piping Design |  |
| - Software should be capable of Automatically route pipelines in 3D space It should also capable of reading fluid diagrams electronically and use the information to interactively create 3D pipelines - Software should be capable of evaluating pipelines for manufacturability and interference |  |
| ##### Reverse Engineering |  |
| - Software should be capable of reading large sets of scanned data containing up to millions of points & should support various industry standard - Software should be capable of filtering the scanned data in order to optimize the cloud point data to prepare digital mockup. It should be capable of automatically fitting digital curves and surfaces to selected raw reference points in the scanned data and gives flexibility to the user to modify the digital data dynamically using various control points - The digital Prototype generated using reverse engineering techniques should be further used for design modifications, generating manufacturing drawings |  |
| ##### Photo-rendering and Animation |  |
| - The software should be able to create photo-realistic images that include effect of shadows, reflection, transparency and texture mapping. - The software should be capable of quickly creating animation of assembly/disassembly with an option of including mechanisms in animation. |  |
| Computer aided Manufacturing (CAM) |  |
| #### Requirement / Capability |  |
| ##### Complete mold design |  |
| - Software should be capable of designing mold cavity geometry with specific functions like automated parting line & parting surface creation, ability to evaluate draft, undercut and thickness as well as projected area, facility to compensate for shrinkage iso-tropically and ansio-tropically, functionality to automatically split the mold into inserts, functionality to create specific features such as runners & Gates. - Software should have library /catalog of all major mold component suppliers – Metric & English - Software should have process driven User Interface for creating customized moldbase with facility of quick preview while developing moldbase It should be able to create moldbase drawings, holecharts & BOM automatically |  |
| ##### Manufacturing Solutions (Lathe /machining canter/EDM) |  |
| - Software should support NC programming for 2.5 axis prismatic milling , up to 5 axis machining ,multi axis turning , mill/turn machining & 4 axis wire EDM - Software should be capable of viewing and managing manufacturing process in a table environment with NC process manager - Software should be capable of Capturing and re-using various machining practices to streamline and standardize manufacturing methodologies. - It should be capable of optimizing finishing tool paths for surface machining |  |
| ##### Manufacturing Solutions (for sheet metal components) |  |
| - Software should support NC programming for turret punch presses, contouring laser/flame machines, nibbling and shearing. - It should support automatic flat pattern development of sheet metal components with bend allowances, automatic punch tool selection and part nesting |  |
| ##### Post Processor Development |  |
| - Software should be capable of developing Post Processors for all Industry standard Machine Tool Controllers. |  |
| Computer Aided Engineering (CAE) |  |
| ##### Structural Thermal & Vibration Analysis |  |
| - Software should be able to perform Static analyses to compute stresses and displacements, including contact non-linear effects, Large deformation non-linearity effects in static analyses, Pre-stress static and pre-stress modal analyses, including spin softening and stress stiffening effects, Modal solutions to compute both free and constrained natural frequencies, Vibration analyses including dynamic frequency, time, random, and shock. In addition it shoul be able to support & perform following capabilities - Buckling analysis capabilities to compute critical buckling load factors , - Steady-state thermal analyses to evaluate a model's reaction to applied thermal loading and boundary constraints , - Non-linear transient thermal analyses to model time-varying thermal events such as the time required for a model to reach a steady state - 2D idealized model types including plane stress, plane strain, 2D plate, 2D unit depth, and axisymmetric - Software should have tools/libraries to quickly build simulation models such as Weld connections, including spot, end, and perimeter; Advanced spring and mass idealizations, Rigid connections, Advanced shells with properties such as laminate lay-ups, Engineering loads and constraints such as bearing loads and cyclic symmetry constraints, Isotropic, orthotropic, and anisotropic materials support, |  |
| ##### Design Optimization |  |
| - Software should be able to perform local as well as global sensitivity studies hat let the engineer evaluate the impact of individual changes on design performance - Software should be able to achieve the optimum model profile based on a combination of design parameters, such as model dimensions and property values - Software should have a generic optimizer interface that allows the use of external design optimization systems |  |
| ##### Fatigue Prediction Tool |  |
| - Software should have capability to predict & improve the fatigue performance. It should be able to calculate result quantities, such as life, damage and factor of safety, It should be able to define and track local and global measures of results quantities for sensitivity and optimization design studies |  |
| ### System Requirements |  |
| - The software should support following Operating systems - Windows XP, Windows 2000. - Linux - HP-US, IRIX & SUN Solaris - The data should be compatible across all the platforms and software should work in a heterogeneous network comprises of various Operating systems like Unix, Linux & Windows XP/2000 |  |
| ### Licensing Requirement |  |
| - The License of the Software should be server based with soft License code - The licensing of the Products should support 100 concurrent uers for each of the functionality as stated in specification A, B, & C - The license (perpetual) for 100 Concurrent users include free updates, upgrades and bug fixing at no extra charge |  |
| Other Terms & Conditions - Vendor should be an ISO 9001: 2000 certified Organization - Vendor should be either developer of the Product or first level Reseller with sufficient experience in sales ( to minimum 30 educational Institutes in India) execution & support of the proposed Product in India. - Vendor must produce list of such 30 Educational Institutes where Vendor has sold the offered product. |  |

D. Equipment B. Tech( Dept. of Electronics & Communication Engineering)

- A.M. Transreceiver kit

Specification: DSB/SSB, Modulator/Demodulator, Amplifier, Filter, Transmitting antennas, Detector, Standard Carrier signal etc.

- F.M. Transreceiver kit

Specification: DSB/SSB, Modulator/Demodulator, Amplifier, Filter, Transmitting antennas, Detector, Standard Carrier signal etc.

- TDM Pulse Amplitude Modulation/Demodulation Training kit

Specification: Crystal oscillator, Analog signal generator, Variable sampling frequency/duty cycle, Filters etc.

- PAM/PPM/PWM Modulation & Demodulation training kit

Specification: Variable sampling/pulse frequency, Voice-commn, Filter, Amplifier/Detector etc.

- ASK/FSK modulation & demodulation Training kit

Specification: On-board data simulator, synchronized 3 carrier sine wave, Test points etc.

6.	PSK & QPSK modulation & demodulation trainer kit

Specification: On-board data simulator, synchronized 3 carrier sine wave, Test points etc.

- Oscilloscope

Specification: (DC-200 MHz, Color/Mono LCD or TFT screen, Dual channel with 4k memory per channel, Built-in mathematical functions, FFT/USB interface, Digital filters, Auto set and calibration functions, Auto measurement etc.)

- Function generator

Specification: Range: 0.2 to 2Mhz, Sine, square, triangular, Pulse, Ramp, TTL output with 20V Vpp

- Multiple Power supply

Specification: variable up to 30 volts, Low ripple, good load/line regulation, short circuit protection etc.

9. 	Delta /Sigma Modulation/Demodulation Trainer Kit:

Specification:

  - Both TX/RX

  - XTL clock

  - Variable frequency generator

  - Variable Gain control

  - Active filter

10.     Data formatting and Carrier Mod/Demod trainer kit:

Specifications:

- Variable carrier gain

- Data inverter

- Various data conditioning formats

- In-phase and Quadrature phase carrier

11.   CDMA/DSSS Trainer Kit :

Specifications:

- Bit error measurement ( upto 10 – 08 )

- Pseudorandom test sequence

- Fast synchronization

- Serial/parallel input selection

12.    Multiplexer/DemUX coder /Decoder trainer Kit:

Specifications:

- Manchester coding/decoding

- Upto 16 channel

- Digital signal generator

List of Instruments for Instrumentation and Control Laboratory

- Electro-Mechanical Transducers Trainer Kit:

Specifications:

Following Linear displacement transducers to be mounted on the test rig:

- Linear variable resistor

- Variable inductor

- Variable area capacitor

- Linear variable differential transducer (LVDT)

- Variable distance capacitor & Strain gauge

- Manual Assignments, covering the Transducers Kits  and including relevant theory and practical information.

- Thermal Transducers Trainer Kit

Specifications:

Following thermal devices and a heat bar assembly with temperature gradient should

be mounted:

- Thermistor

- Thermal reed relay

- Platinum resistance

- Bi-metallic switch n Thermocouple

- Manual Assignments, covering the Transducers Kits and including relevant theory and practical information.

3. Optical Transducers Trainer Kit

Specifications:

A light source and the following sensors should be mounted:

- Photoconductive cell

- Photodiode n Phototransistor

- Manual Assignments, covering the Transducers Kits and including relevant theory and practical information.

4. Sensor Signal processing Trainer kit :

Specifications:

Following instrumentation  and signal conditioning circuitry should comprise:

- Wheatstone Bridge

- Oscillator

- Operational Amplifier

- Discriminator  Power Amplifier

  - Servo Control Trainer Kit:

Specifications:

The system should consists of three main units:

Mechanical Unit: Input and output potentiometers, motor, tacho-generator,

absolute and incremental encoder and waveform generators.

Signal Unit: Error amplifier, Optional additional time constant.

Variable gain  and velocity feedback. Single, two term or full PID control.

Digital Unit: Encoders should provides access to PC interfacing with ADC &

DA linear or   PWM motor drive, LED display. Feedback   software supports for

both the Analogue and Digital units.

6. DC Servo Control Trainer Kit :

Specifications:

Analogue Control, Digital Control unit and the Mechanical unit.

DC Power Supply, interface cables, software. PC-based instrumentation, recorder.

7.   AC Servo Control Trainer Kit :

Specifications:

- Differential Synchro

- Sample and Hold

- Proportional, Integral and Derivative Unit

- Simulated Relay

- Relay characteristics and relay operated systems

- Effect of backlash on the system

- Phase-plane analysis

- PID control loop characteristics

- Speed control

- Position control

- Waveform sampling

- Sampled data servo control

- Simulated sampled data control

- Sampled data process control

- Differential sync

  - Multifunctional DAQ Card.

Specification:

    - USB compatible.

    - LABView compatible driver

    - 12-bit resolution, 100KS/s sampling rate.

    - Unipolar / Bipolar Input range

    - High Gain

    - 4K samples FIFO for A1

    - Minimum 2-ch AO

    - Minimum 16 DI/ 16DO/ 1CRT

    - Board I/O

    - Complete with all connectors and accessories.

  - Multifunctional DAQ Card.

Specification:

i) USB compatible

LabView compatible driver

- 12-bit A/D 100 KS/s sampling rate

- 16 S.E. AI

- Programmable Gain for each AI

- Automatic channel /Gain scamming

- On Board 1 K samples FIFO buffer

- Two 12 bit AO

- 16 DI and 16 DO

- Programmable Pacer/ Control

- Complete with all connectors and Accessories

  - Multifunctional DAQ Card

Specification:

    - USB compatible

    - LABView compatible driver

    - 12-bit resolution, 1 MS/s sampling rate.

    - PCI bus mastering for data transfer

    - Auto calibration function

    - 7 data acquisition modes for AI function.

    - 16 AI channel with 1K FIFO

    - 2 AO / 16 DIO/ 3CRT

    - 32K FIFO for real time wave output function

    - Complete with all connectors and accessories

  - Four axis stepping/ Pulse type Motor control card.

Specification:

    - USB compatible

    - LABView compatible driver

    - Hand wheel and jog function

    - 3 axis linear interpolation, 2 axis circular interpolation and continuous interpolation

    - Programmable T/S curve acceleration/ deceleration rate

    - Up to 4 MPS pulse output for each axis

    - Position management and S/W limit switch function

    - Constant speed control

    - Board ID

    - Complete with all connectors and accessories

    - A compatible stepper motor.

12. Four axis Voltage Type Servo Motor Control Card.

Specification:

- USB compatible

- LABView compatible driver

- 4 axis servo positioning control

- 4 ch 16 bit D/A converter

- 6 ch 12 bit D/D converter

- 5 ch encoder input

- 13 dedicated input and 5 dedicated output

- 256 remote serial input/ output interfaces

- Complete with all connectors and accessories

- A compatible servo motor

13. DAS module with programmable Gain/ Programmable High Gain.

Specification:

- USB compatible

- LABView compatible driver

- 16 signal ended or 8 differential analog inputs

- 12 bit A/D converter

- Two 8 bit digital input/ output channels, TTL compatible

- Complete with all connectors and accessories

14. Isolated Thermocouple input module

Specification

  - USB compatible

  - LABView compatible driver

  - Switch configurable J. K. T. R. S. E. B type

  - Thermocouple inputs

  - Voltage output : 0-10V

  - Complete with all connectors and accessories

  - A compatible thermocouple

15. Isolated RTD Input module

Specification:

- USB compatible

- LABView compatible driver

- Switch configurable Pt and NI RTD input ranges

- Switch configurable for 0-5V or 0-20mA outputs

- Complete with all connectors and accessories

- A compatible RTD

16. Isolated Strain Gauge Input module

Specification:

- USB compatible

- LABView compatible driver

- Switch configurable mV or V input of full bridge

- Switch configurable voltage and current outputs

- 1000 Vdc 3 way optical isolation

- Complete with all connectors and accessories

- A complete load cell

17. AC voltage sensor module

Specification:

- USB compatible

- LABView compatible driver

- 0-450 Vac to 0-5Vdc

- Input inpedent : 2M

- Accuracy: + 0.1% FSR

- Complete with all connectors and accessories

18. AC current input module

Specification:

- USB compatible

- LABView compatible driver

- 0-5 Vac to 0-5 Vdc

- Input inpedent: 2M

- Accuracy : + 0.1% FSR

- Complete with all connectors and accessories

List of Accessories for Machine Laboratory B Tech (ECE)

- Rheostat 50 Ohm, 10A

- Rheostat 500 Ohm, 2 A

- Shunt:

a) 5 A

b) 10 A

c) 15 A

- Tachometer : Digital display contact type

- Tachometer: Mechanical type

- Resistive Load Bank

a) 5 KW, 1 Ph, 220 V (variable switch type)

b) 5 KW, 3 Ph, 415 V (Variac Type)

- Portable Ammeter: (table top type)

  - 0-10 A

  - 0-2 A

- Panel Meter:

Ammeter: 10A, 20A,30A

Voltmeter : 250V,500V

- Copper wire:

  - 7/20

  - 7/18

  - 7/22

  - 23/18

- Knief Switch( 10A,15A,20A)

  - SPST

  - DPST

  - DPDT

## GENERAL TERMS & CONDITIONS

- No separate tender paper will be issued from the office; one should only download the specifications from the web site.

- The rates should be quoted against each item as shown in the list in the same chronological order along with specifications and supporting documents (technical details and brochures etc.).

- Quotation for each department should be in separate cover.

- Quotations should be accompanied by-

i. A call deposit (earnest money) amounting to Rs. 5,000/- pledge in favour of

the Registrar, Tezpur University, payable at Tezpur.

- The rates should be exclusive of taxes and applicable tax % should be clearly indicated.

- At least one year warranty for all equipment should be provided.

- The University is exempted from Custom & Excise duty.

- Terms and Conditions of supply and installation (if required) should be clearly indicated in the quotations.

- The supplier should be in a position to deliver and install the ordered items within the stipulated time mentioned in the supply order.

- Items must be delivered at Napaam campus of the University at the cost of the supplier.

- The rates should be valid at least for a period of one year.

- The University reserves the right to accept or reject any or all the quotations without assigning any reason.

- Copies of Up-to-date VAT clearance certificates, indicating TIN number etc. should be enclosed.

- Copy of Manufacturer/ Distributorship/ Dealership Certificate has to be enclosed with the offer.

- Bidders should clearly state the existing after Sales Service & Support Network and Customer base in North-East India.

- Rates quoted should be on F.O.R. Tezpur University, Tezpur basis with following breakups-

Basic Price

(+) Central Excise Duty(CED)

(+) Packaging & Forwarding Charges(P&F) (if any)

Sub-total Price (Including CED and P&F Charges)

(+) VAT/CST (Sub-total Price (Including CED and P&F Charges)

Sub-total Price (Including CED +P&F+VAT/CST)

(+)Freight & Insurance Charge(If any)

(+) Installation & Commissioning Charge(including Service Tax)(if any)

Grand Total*(F.O.R. Tezpur University) Price

(*Assam Govt. Entry Tax @4% on the total Invoice Value is required to be paid by this Institute on CST Billed items)

- Applications should be forwarded to the Registrar for release of call deposit against unaccepted quotations.

***